Experiment 6 :

Objective : Develop a simulation to demonstrate multiple access protocols such as Pure ALOHA, Slotted ALOHA, CSMA/CD, and CSMA/CA. Analyze the performance of each protocol in handling network collisions and maximizing data transmission efficiency

Components required :

Hardware Components (in Cisco Packet Tracer):

- Components for CSMA/CD (Wired Ethernet)

Required Devices:

- PCs

- Hub (important – collision domain)

- Copper Straight-through Cables

2. Components for CSMA/CA (Wireless Network)

Required Devices:

- Laptop

- Access Point

- Wireless NIC (built-in in laptops)

CSMA/CD (Collision Detection)

Collision Handling:

- Device checks channel before sending

- If two devices send together → collision happens

- Collision is detected instantly

- Transmission stops and jam signal is sent

- Devices wait random time, then resend

Performance:

- Works well when network traffic is low

- Performance decreases as devices increase

- More collisions → more delays

Efficiency:

- Good efficiency in small networks

- Poor efficiency in heavy traffic

- Bandwidth gets wasted due to retransmissions

CSMA/CA (Collision Avoidance)

Collision Handling:

- Device checks channel before sending

- Waits for a random backoff time

- Uses RTS/CTS signals to avoid collision

- Collisions are minimized before they happen

Performance:

- Suitable for wireless networks

- Stable even when traffic increases

- Slight delay due to waiting mechanism

Efficiency:

- Better efficiency than CSMA/CD in busy networks

- Less data loss due to fewer collisions

- Some bandwidth used for control signals

Conclusion:

- CSMA/CD is simple but not efficient for large or busy networks

- CSMA/CA provides better performance by reducing collisions, especially in wireless networks

CSMA/CD:

Steps:

  - Place hub in centre

  - Place 4 PCs around hub

  - Connect all PCs to the Hub using Copper Straight-Through Cable

  - Click each PC → Desktop → IP Configuration

Assign IPs in same network:

PC1 → 192.168.1.1

PC2 → 192.168.1.2

PC3 → 192.168.1.3

PC4 → 192.168.1.4

  - Click Add Simple PDU (envelope icon)

  - Send data:

From PC1 → PC2

Quickly send another from PC3 → PC2

This creates simultaneous transmission → collision

CSMA/CA:

Steps:

1. Place Devices

- Drag and drop:

  - Access Point

  - 4 Laptop

2. Configure Wireless Network

- Click Access Point → Config

- Set:

  - SSID (e.g., MyWiFi)

  - Security (optional, e.g., WPA2)

3. Connect Wireless Devices

- Click Laptop → Desktop → PC Wireless

- Select your SSID → Connect

4. Assign IP Addresses

- Desktop → IP Configuration

- Use: DHCP (automatic)

5. Switch to Simulation Mode

- Click Simulation Mode

6. Generate Traffic

- Use Add Simple PDU (envelope icon)

- Send data between laptops